- E10-M4463D-SMA Wireless module

Description:

E10-M4463D-SMA is a middle power 433MHz frequency wireless transceiver module. It has features of high performance, long distance and high air rate. It adopts original SI4463-B1B or SI4438-B1C chip, which is industrial.

Specification:

| NO. | Parameter Name | Parameter Details |
| --- | --- | --- |
| 1 | Module size | 16.6 * 24.1 mm |
| 2 | source | all imported，origin : United States, Japan |
| 3 | Production | machine stickers |
| 4 | Interface | 1.27mm SMD type, you can use the universal plate and DuPont line |
| 5 | Supply Voltage | 1.8V-3.6VDC |
| 6 | communication level | 0.7VDD-3.6VDC，VDD is modules’voltage |
| 7 | measured distance | 2100m@1K，condition：city,in open area,30℃,cloudy |
| 8 | Maximum Power | 20dBm，about 100mW |
| 9 | air rate | 0.123K - 1MBPS (adjustment via software) |
| 10 | shutdown current | About 30nA，test conditions：PD mode，SDN=1 |
| 11 | Power Level | -20 - +20dBmadjustable |
| 12 | Transmission current | maximum peak is 80mA |
| 13 | Receiving current | 10MA，condition：3.0VDC power supply |
| 14 | Antenna type | 433M spring antenna/SMA built-in (_optional) |
| 15 | interface | Standard. (SPI)0，maximum 10Mbps air rate |
| 16 | Transmission length | Single data 1-64 bytes , supports dynamic length |
| 17 | Receiving length | Single data 1-64 bytes , supports dynamic length |
| 18 | RSSI | support |
| 19 | Receiving sensitivity | -121dBm |
| 20 | Working temperature | -30 - +85℃ |
| 21 | Working humidity | <90% relative humidity |
| 22 | Storage temperature | -40 - +125℃ |
| 23 | Working frequency | 425MHz - 525MHz，software adjustable（module oscillator is 30MHz） |

Application:

This module can be widely applied in high-speed wireless data transmission, remote acquisition, security alarm, power monitoring, intelligent household, robots, model aircraft , lighting control, intelligent buildings, medical equipment, wireless audio, VOIP applications, high-end toys, intelligent sports equipment, wireless sensor networks.

Pin:

Pin Definition:

| pin | Input/output | Description |
| --- | --- | --- |
| GND |  | ground |
| SDN |  | pin，open in low level， |
| GPIO3 | output | Module data putput pin |
| GPIO2 | output | Module data output pin |
| nSEL | 输入 | SPI chip select pin，valid in low level |
| MOSI | 输入 | SPI data input pin |
| MISO | output | SPI data output pin |
| SCK | output | SPI clock pin |
| IRQ | output | Module interrupt pin |
| GPIO1 | output | Module data output pin |
| GPIO0 | output | Module data output pin |
| VDD |  | Power supply pin，1.8V-3.6V |

Notice:

1,Analog Devices has a high frequency electrostatic sensitive nature , avoid human contact module of electronic components as possible . Our production process is fully implemented in accordance with the higher ESD standards.

2,When welding , soldering iron needs good ground , on request when mass production staff wear a grounded wrist strap cable .

3, greater impact on power quality module performance , please ensure that the power supply module has a small ripple , be sure to avoid frequent power dramatically shake . Recommend the use of π -type filter ( tantalum capacitor + inductor ) .

4,module using single-point ground grounding is recommended to use 0 ohm resistor , inductor or 10mH , separate from other parts of the circuit ground reference.

5,module antenna mounting structure has a greater impact on module performance , be sure that the antenna exposed , the best vertical. When the module is installed in the chassis interior , you can use high-quality antenna extension cable will extend the antenna to the outside of the case . Antenna must not be installed inside the metal shell , will lead to the transmission distance significantly weakened.

6,same product if there are other internal band wireless devices , due to the possibility harmonic interference , please increase as much as possible with the linear distance between the module and the possible use of metal materials to separate the two .

7,If the existence of a crystal near the location of the module circuit board, increasing as much as possible with the linear distance between the crystal , quartz crystal as far as possible with a metal shell package , crystal wiring should be a " floor " approach to the parcel.

## How to use:

The single chip micromputer connect to SCK、MISO、MOSI、nSEL，the rest GPIO0-GPIO1can be used tp output data. GPIO2 and GPIO3 must be setted into special mode, whichthe single chip micromputer can’t control.

## Common problems:

### Communication distance is too short

- 433 m frequency band because of its physical properties When linear communication obstacles, communication distance will shorten different barrier properties, the attenuation degree of inconsistency

- Temperature and humidity, co-channel interference, packet loss rate will lead to improved communication.

- There are metal objects near the antenna, will absorb part of the wireless signal.

- Register power setting, rate setting. The faster the rate, the closer.

- Antenna can not be placed in a metal case, the decay is very serious.

- Module fever, easily damaged.

- Check the power supply to ensure that between 1.8V-3.6V.

- Check the power supply stability, does not significantly fluctuate frequently.

- Ensure the installation process antistatic operation. High-frequency electrostatic   sensitive devices.

- Communication lines can not use 5V level.