Unidarts N2 kit manual

If you have any questions during installation, please contact me via Facebook Messenger

https://www.facebook.com/codebluelsj

Installation (please follow the YouTube video)

Galaxy II / common               Spider 1K/2K

N2 Chip Working Logic

Default Boot Mode
The N2 chip boots into Bluetooth (BT) mode by default, indicated by two beeps.

Switching to Bypass Mode
While in BT mode, press BTN + OB simultaneously to temporarily switch to Bypass mode. You will hear three beeps indicating the mode switch.

Bypass Mode Functionality
In Bypass mode, the original system can be used as normal.

Switching Back to Bluetooth Mode
To return to BT mode from Bypass mode, simply reconnect to the chip using the PREVIOUS app. The chip will automatically switch back to BT mode.
If the app is already connected, disconnect and reconnect. Hearing two beeps confirms the switch back to BT mode.

Shortcuts:  (only works in BT mode, BTN stands for player change button)

HOLD the player change button for at least 1 second.

BTN + OB  =  switch to BYPASS mode temporary.

BTN + IB   =  perform OTA UPDATE

BTN + D3  =  switch board size to 13inch/15inch ( !!! G3 only)

- BTN + Inner S1 = GRANBOARD

- BTN + Inner S2 = DARTSLIVE HOME

- BTN + Inner S3 = VDarts Game

- BTN + Inner S4 = SDB Play 2

- BTN + Inner S5 = DARTSBEAT HOME

- BTN + Inner S6 = Guz World

- BTN + Inner S7 = Unicorn Score Buddy

- BTN + Inner S8 = DR-DARTS

- BTN + Inner S9 = ADARTS

Firmware upgrade

If you encounter a firmware bug, you can solve it through online upgrade.

1. Prepare a mobile WiFi hotspot named unidarts and password 12345678

In BT mode, press and hold player change button + the inner bull to trigger the upgrade process.

2. There will be a beep~beep~beep prompt during the upgrade process. After the upgrade is completed, there will be a long and two short beeps.

3. Then restart and enter BT mode, you can use the APP to connect, and refer to the above application switching steps to switch different applications.

Dart machine type switching.

With Ver 2.1 PCB and above, you should able to find a blue “MODE SW” encoder. You can rotate the encoder to point to proper number, and reset the chip, you can tell from the screen what mode it boots into.

[1] G1/KDS    [2] GALAXY G2/2.5     [3] Spider 360

[4] Valley/Lowen   [5] SIPULS     [6] CYBERDINE

[7] MEDALIST/Scorpion   [8] ENTEREAL      [9] Compumatic

[10] Radikal

FAQ:

Q: Why the chip plays on itself with random numbers.

A: Please try another power supply.

Q: How to adjust the sensitivity of the miss sensor

A: Change to BT mode, switch to GRANBOARD, and connect with the APP. Go to APP settings > board > lower the “out area reaction sensitivity”. once you done that, it will take effect with all APPs.

Q: Why some of my darts not register.

A: try throw it slowly ( 1s between each dart).  if that works fine. Please change the board response interval.  Change to BT mode, switch to GRANBOARD, and connect with the APP. Go to APP settings > board > shorten the “board response interval”

Interval = (n + 2) x 100ms.   Example: Number 4 = 600ms