SKU List

A:Maker-esp32-pro

B:Maker-esp32-pro + Building Block Shell

C:Maker-esp32-pro + 18650 Battery Module (including battery) + Four Encoder Motors

D:Maker-esp32-pro Main Board + 18650 Battery Module (including battery) + Four Motors

E:Maker-esp32-pro Main Board + 18650 Battery Module (Including Battery)

F:Maker-esp32-pro Main Board + PS3 Gamepad

G:Maker-esp32-pro Main Board + PS3 Gamepad + 18650 Battery Module (Excluding Battery)

H:Maker-esp32-pro Main Board + PS3 Gamepad + 18650 Battery Module (Including Battery)

I:Maker-esp32-pro Main Board + PS3 Gamepad + 18650 Battery Module (Including Battery) + Four Encoder Motors

J:Maker-esp32-pro Main Board + PS3 Gamepad + 18650 Battery Module (Including Battery) + Four Motors

Brief introduction

MAKER-ESP32-PRO is a signature product for maker education developed based on Espressif Systems' ESP32-WROOM-32 module, featuring 4MB Flash and a single-chip solution integrating 2.4 GHz Wi-Fi + Bluetooth dual-mode.

Onboard, it supports 4-channel encoder motors and DC motors with a maximum driving current of 3A. The encoder/DC motors share the same driver pins—the same port cannot drive encoder and DC motors simultaneously.

Technical specifications

Specifications of the Module Used in MAKER-ESP32-PRO

- 448 KB ROM, 520 KB SRAM, 16 KB RTC SRAM

- QSPI supports multiple flash/SRAM.

- Built - in 8 MHz oscillator.

- Supports self - calibration.

- Built - in RC oscillator, supporting self - calibration.

- Supports an external main crystal oscillator ranging from 2 MHz to 60 MHz (Currently, only a 40 MHz crystal oscillator is supported if Wi - Fi/Bluetooth functions are used).

- Supports an external 32 kHz crystal oscillator for the RTC, supporting self - calibration.

- Two timer groups, each group including two 64 - bit general - purpose timers and one main system watchdog.

- One RTC timer.

- RTC watchdog.

- 34 GPIO ports.

- 12 - bit SAR ADC with up to 18 channels.

- Ethernet MAC interface with dedicated DMA, supporting IEEE 1588.

- Two - wire automotive interface (TWAI®, compliant with ISO11898 - 1).

- IR (TX/RX)

- MAKER-ESP32-PRO specifications and specification comparisonFour encoder motor ports (E0, E1, E2, E3); four DC motor ports (M0, M1, M2, M3), which share the same ports with the encoder motors. The maximum current can reach 3A.

- Among them, E0 (A: 18, B: 19, +: 27, -: 13), E1 (A: 5, B: 23, +: 4, -: 2), E2 (A: 35, B: 36, +: 17, -: 12), E3 (A: 34, B: 39, +: 15, -: 14).

- Five I2C pin headers and one SPI pin header.

- Four servo interfaces (25, 26, 32, 33).

- Thirteen IO pins (5, 12, 14, 15, 16, 17, 18, 19, 23, 34, 35, 36, 39).

- Input voltage: 6 - 16V.

- Product dimensions: 80mm × 57mm; PCB thickness: 1.6mm; Net weight: 35g.

- Diameter of M4 locating holes: 4.6mm, compatible with Lego.

- Software support: Mixly, Arduino IDE, Python.

Experimental scenario

- Compatible with PS3 Gamepad